# MODULE 1: COMPUTER FUNDAMENTALS & PROGRAMMING CONCEPTS

## Days 1-3 | 12 Hours (6 Hours Theory + 6 Hours Practical)

### Reference: Foundations of Computing — Pradeep Sinha & Priti Sinha

# DAY 1: Computer Evolution, Architecture & Number Systems

## THEORY (2 Hours)

### 1.1 Generations of Computers

Computers have evolved through five distinct generations, each marked by a major technological advancement:

First Generation (1940–1956): Vacuum Tubes - Technology: Vacuum tubes for circuitry, magnetic drums for memory - Characteristics: Enormous size (filled entire rooms), very expensive, generated tremendous heat, unreliable, consumed huge electricity - Programming: Machine language only (binary 0s and 1s) - Examples: ENIAC (Electronic Numerical Integrator and Computer), UNIVAC I, IBM 701 - ENIAC weighed 30 tons and had 18,000 vacuum tubes

Second Generation (1956–1963): Transistors - Technology: Transistors replaced vacuum tubes - Characteristics: Smaller, faster, cheaper, more reliable, less heat - Programming: Assembly language, early high-level languages (FORTRAN, COBOL) - Memory: Magnetic core memory - Examples: IBM 1401, IBM 7094, CDC 1604

Third Generation (1964–1971): Integrated Circuits (ICs) - Technology: ICs (many transistors on a single silicon chip) - Characteristics: Even smaller, faster, more reliable, keyboard and monitor introduced - Programming: High-level languages (BASIC, Pascal, C) - OS: Time-sharing operating systems introduced - Examples: IBM 360 series, PDP-8, PDP-11

Fourth Generation (1971–Present): Microprocessors - Technology: VLSI (Very Large Scale Integration) — thousands/millions of transistors on one chip - The microprocessor: entire CPU on a single chip - Intel 4004 (1971): first microprocessor - Characteristics: Personal computers became possible, affordable, GUI introduced - Examples: IBM PC, Apple Macintosh, all modern PCs - Intel processors: 4004 → 8008 → 8086 → 80286 → 80386 → Pentium → Core series

Fifth Generation (Present & Beyond): Artificial Intelligence - Technology: ULSI, parallel processing, AI-based computing - Characteristics: Natural language processing, machine learning, voice recognition, expert systems - Goal: Computers that can think, learn, and make decisions - Examples: IBM Watson, Google DeepMind, modern AI assistants

C-CAT Tip: Questions typically ask “Which generation used [technology]?” or “ENIAC belongs to which generation?” Memorize the technology-to-generation mapping!

### 1.2 Classification of Computers

By Size and Capability:

| Type | Characteristics | Examples |
| --- | --- | --- |
| Microcomputer | Single user, single microprocessor, cheapest | Desktop PC, Laptop, Tablet |
| Minicomputer | Multi-user (up to 200), mid-range | PDP-11, IBM AS/400, VAX |
| Mainframe | Large, high-speed, multi-user (thousands), very expensive | IBM zSeries, System z |
| Supercomputer | Fastest, parallel processing, used for scientific computation | PARAM (India), Fugaku (Japan), Frontier (USA) |

By Purpose: - General Purpose: Can perform various tasks (PCs, laptops) - Special Purpose: Designed for specific task (ATM, washing machine controller)

By Data Handling: - Analog: Process continuous data (speedometer, thermometer) - Digital: Process discrete data (binary 0/1) — all modern computers - Hybrid: Combination (hospital ICU monitors — analog sensors + digital processing)

### 1.3 Basic Computer Organization

Every computer follows the IPO (Input-Processing-Output) cycle based on Von Neumann Architecture:

Input Devices → CPU (Processing) → Output Devices
                    ↕
               Memory (Storage)

Input Devices: Keyboard, Mouse, Scanner, Microphone, Webcam, Joystick, Barcode reader, Touch screen Output Devices: Monitor, Printer, Speaker, Projector, Plotter Both Input & Output: Touch screen, Modem, Network card

### 1.4 CPU Architecture

The Central Processing Unit (CPU) is the brain of the computer with three main components:

1. Arithmetic Logic Unit (ALU) - Performs all arithmetic operations (+, -, ×, ÷) - Performs all logical operations (AND, OR, NOT, comparison) - Contains the Accumulator — a register that stores intermediate results

2. Control Unit (CU) - Directs and coordinates all computer operations - Fetches instructions from memory, decodes them, executes them - Controls data flow between CPU, memory, and I/O devices - Does NOT process data — only controls the flow

3. Registers - Small, ultra-fast storage locations inside CPU - Types: - Program Counter (PC): Holds address of next instruction - Instruction Register (IR): Holds current instruction being executed - Accumulator (ACC): Stores intermediate results - Memory Address Register (MAR): Holds address of memory location being accessed - Memory Data Register (MDR): Holds data being transferred to/from memory

System Bus — the communication highway between components: - Address Bus: Carries memory addresses (unidirectional: CPU → Memory) - Data Bus: Carries actual data (bidirectional: CPU ↔ Memory) - Control Bus: Carries control signals (bidirectional)

C-CAT Tip: “CU does not process data” and “ALU performs both arithmetic AND logical operations” are frequently tested facts.

### 1.5 Memory Hierarchy

Memory is organized in a hierarchy based on speed, cost, and capacity:

Registers (fastest, smallest, most expensive per bit)
    ↓
Cache Memory (L1, L2, L3 — SRAM, very fast)
    ↓
Main Memory / RAM (DRAM, fast, volatile)
    ↓
Secondary Storage (HDD, SSD — slow, non-volatile, cheapest per bit)

Key Concepts: - Volatile memory: Loses data when power is off (RAM, Cache, Registers) - Non-volatile memory: Retains data without power (ROM, HDD, SSD, Flash) - Cache memory: Small, fast memory between CPU and RAM; stores frequently accessed data - L1 Cache: Smallest, fastest, inside CPU core - L2 Cache: Larger, slightly slower, per core or shared - L3 Cache: Largest, shared among all cores

### 1.6 Number Systems

This is the MOST TESTED computer fundamentals topic in C-CAT. Master this completely!

Four Number Systems:

| System | Base | Digits Used | Example |
| --- | --- | --- | --- |
| Binary | 2 | 0, 1 | 1010₂ |
| Octal | 8 | 0-7 | 752₈ |
| Decimal | 10 | 0-9 | 459₁₀ |
| Hexadecimal | 16 | 0-9, A-F (A=10, B=11, C=12, D=13, E=14, F=15) | 2AF₁₆ |

### 1.6.1 Decimal to Binary Conversion

Method: Repeatedly divide by 2, note remainders (read bottom to top)

Example: Convert 45₁₀ to Binary

45 ÷ 2 = 22  remainder 1  ↑
22 ÷ 2 = 11  remainder 0  |
11 ÷ 2 = 5   remainder 1  |  Read upward
5  ÷ 2 = 2   remainder 1  |
2  ÷ 2 = 1   remainder 0  |
1  ÷ 2 = 0   remainder 1  |

Answer: 45₁₀ = 101101₂

For fractional part: Multiply by 2, note integer parts (read top to bottom)

Example: Convert 0.625₁₀ to Binary

0.625 × 2 = 1.250  → 1  ↓
0.250 × 2 = 0.500  → 0  |  Read downward
0.500 × 2 = 1.000  → 1  ↓

Answer: 0.625₁₀ = 0.101₂

### 1.6.2 Binary to Decimal Conversion

Method: Multiply each digit by 2^(position) and sum (position starts from 0, right to left)

Example: Convert 101101₂ to Decimal

1×2⁵ + 0×2⁴ + 1×2³ + 1×2² + 0×2¹ + 1×2⁰
= 32 + 0 + 8 + 4 + 0 + 1
= 45₁₀

### 1.6.3 Decimal to Octal Conversion

Method: Repeatedly divide by 8

Example: Convert 247₁₀ to Octal

247 ÷ 8 = 30  remainder 7  ↑
30  ÷ 8 = 3   remainder 6  |
3   ÷ 8 = 0   remainder 3  |

Answer: 247₁₀ = 367₈

### 1.6.4 Decimal to Hexadecimal Conversion

Method: Repeatedly divide by 16

Example: Convert 431₁₀ to Hexadecimal

431 ÷ 16 = 26  remainder 15 (= F)  ↑
26  ÷ 16 = 1   remainder 10 (= A)  |
1   ÷ 16 = 0   remainder 1         |

Answer: 431₁₀ = 1AF₁₆

### 1.6.5 Binary ↔ Octal (Quick Method)

Binary to Octal: Group binary digits in sets of 3 (from right), convert each group

101 101₂ = ?₈
101 = 5, 101 = 5
Answer: 55₈

Octal to Binary: Convert each octal digit to 3-bit binary

367₈ = ?₂
3 = 011, 6 = 110, 7 = 111
Answer: 011110111₂ = 11110111₂

### 1.6.6 Binary ↔ Hexadecimal (Quick Method)

Binary to Hex: Group binary digits in sets of 4 (from right), convert each group

0110 1111₂ = ?₁₆
0110 = 6, 1111 = F
Answer: 6F₁₆

Hex to Binary: Convert each hex digit to 4-bit binary

2AF₁₆ = ?₂
2 = 0010, A = 1010, F = 1111
Answer: 001010101111₂

### 1.7 Binary Arithmetic

Binary Addition Rules:

0 + 0 = 0
0 + 1 = 1
1 + 0 = 1
1 + 1 = 10 (0, carry 1)
1 + 1 + 1 = 11 (1, carry 1)

Example: 1011 + 1101

1 0 1 1
+ 1 1 0 1
---------
1 1 0 0 0

### 1.8 Complements: 1’s and 2’s Complement

1’s Complement: Flip all bits (0→1, 1→0)

1's complement of 01010101 = 10101010

2’s Complement: 1’s complement + 1

2's complement of 01010101:
Step 1 (1's complement): 10101010
Step 2 (add 1):        + 00000001
                        ----------
                         10101011

Why 2’s complement matters: It is used to represent negative numbers in computers. - Positive numbers: stored as-is in binary - Negative numbers: stored as 2’s complement - For an 8-bit number: range is -128 to +127

Example: Represent -5 in 8-bit 2’s complement

+5 in binary:    00000101
1's complement:  11111010
Add 1:          +00000001
                ----------
2's complement:  11111011  ← This is how -5 is stored!

Binary Subtraction using 2’s complement: A - B = A + (2’s complement of B)

Example: 7 - 3 (in 4-bit)

7 = 0111
3 = 0011, 2's complement of 3 = 1100 + 1 = 1101
0111 + 1101 = 10100 → Discard carry → 0100 = 4₁₀ ✓

### 1.9 BCD (Binary Coded Decimal)

Each decimal digit is represented by its 4-bit binary equivalent:

Decimal 59 in BCD:
5 → 0101, 9 → 1001
BCD: 0101 1001

C-CAT Trap: BCD of 59 is NOT the same as binary of 59! Binary of 59 = 111011 (different from BCD 01011001)

## MCQs — DAY 1 (20 Questions)

Q1. ENIAC computer belongs to which generation? - A) Second Generation - B) First Generation - C) Third Generation - D) Fourth Generation

Answer: B) First Generation Explanation: ENIAC (1945) used vacuum tubes, which is the defining technology of the first generation. Distractor A (Second Gen) is tempting because students sometimes confuse ENIAC with transistor-era computers.

Q2. Which technology was used in the Third Generation of computers? - A) Vacuum Tubes - B) Transistors - C) Integrated Circuits - D) Microprocessors

Answer: C) Integrated Circuits Explanation: 3rd Gen (1964-1971) used ICs. Distractor D (Microprocessors) is tempting — microprocessors came in the 4th generation.

Q3. The brain of the computer is: - A) Memory - B) Control Unit - C) CPU - D) ALU

Answer: C) CPU Explanation: CPU is the brain. Distractor B (Control Unit) is tempting because CU controls operations, but CU is a PART of the CPU, not the brain itself. The CPU includes ALU + CU + Registers.

Q4. Which of the following is NOT a function of the Control Unit? - A) Fetching instructions from memory - B) Performing arithmetic calculations - C) Decoding instructions - D) Controlling data flow

Answer: B) Performing arithmetic calculations Explanation: Arithmetic calculations are performed by ALU, not CU. CU only directs and coordinates. This is a frequently tested distinction.

Q5. Convert 110101₂ to decimal: - A) (select the correct one) - B) 53 - C) 51 - D) 55

Answer: B) 53 Calculation: 1×2⁵ + 1×2⁴ + 0×2³ + 1×2² + 0×2¹ + 1×2⁰ = 32 + 16 + 0 + 4 + 0 + 1 = 53. Distractor C (51) results from miscalculating 2⁴ as 14 instead of 16.

Q6. The decimal equivalent of hexadecimal number 2A is: - A) 42 - B) 32 - C) 44 - D) 50

Answer: A) 42 Calculation: 2×16¹ + A×16⁰ = 32 + 10 = 42. Distractor B (32) is what you get if you forget to add the A (10) value — a common error.

Q7. Convert 255₁₀ to binary: - A) 11111110 - B) 11111111 - C) 10000000 - D) 11111100

Answer: B) 11111111 Calculation: 255 = 128+64+32+16+8+4+2+1 = all eight 1s. Distractor A (11111110) = 254, a common off-by-one error.

Q8. The octal equivalent of binary number 110111₂ is: - A) 57₈ - B) 67₈ - C) 37₈ - D) 47₈

Answer: B) 67₈ Method: Group in 3s from right: 110 111 → 6 7 → 67₈. Distractor A (57) results from incorrectly grouping as 101 111.

Q9. The 2’s complement of 8-bit number 00110100 is: - A) 11001011 - B) 11001100 - C) 11001010 - D) 11001101

Answer: B) 11001100 Step 1: 1’s complement = 11001011. Step 2: Add 1 → 11001100. Distractor A (11001011) is the 1’s complement — students often stop at step 1 and forget to add 1!

Q10. The hexadecimal equivalent of binary 10110110₂ is: - A) A6₁₆ - B) B6₁₆ - C) B5₁₆ - D) C6₁₆

Answer: B) B6₁₆ Group in 4s: 1011 0110 → B(11) 6 → B6₁₆. Distractor A (A6) results from misreading 1011 as A(10) instead of B(11).

Q11. Which memory is volatile? - A) ROM - B) Hard Disk - C) RAM - D) Flash Memory

Answer: C) RAM Explanation: RAM loses data when power is off. All other options are non-volatile. This is a basic but frequently tested fact.

Q12. The BCD representation of decimal 83 is: - A) 10000011 - B) 1000 0011 - C) 1010011 - D) 01010011

Answer: B) 1000 0011 8 → 1000, 3 → 0011 → 1000 0011. Distractor A looks the same but without the space/grouping. Distractor D (01010011) is the actual binary of 83, NOT BCD — this is the classic C-CAT trap!

Q13. In the memory hierarchy, which is the fastest? - A) RAM - B) Cache - C) Registers - D) SSD

Answer: C) Registers Registers are inside the CPU and are the fastest. Cache is second fastest. Distractor B (Cache) is tempting because cache is often discussed as “fast memory” — but registers are faster.

Q14. The address bus is: - A) Bidirectional - B) Unidirectional (CPU to Memory) - C) Unidirectional (Memory to CPU) - D) Not connected to CPU

Answer: B) Unidirectional (CPU to Memory) The address bus carries addresses FROM CPU TO memory. The DATA bus is bidirectional. Students often confuse the two.

Q15. Convert 0.75₁₀ to binary: - A) 0.10 - B) 0.01 - C) 0.11 - D) 0.101

Answer: C) 0.11 0.75 × 2 = 1.50 → 1; 0.50 × 2 = 1.00 → 1. Result: 0.11₂. Verify: 0.5 + 0.25 = 0.75 ✓. Distractor D (0.101) = 0.625, not 0.75.

Q16. How many unique values can be represented by 8 bits? - A) 128 - B) 255 - C) 256 - D) 512

Answer: C) 256 2⁸ = 256 (values from 0 to 255). Distractor B (255) is the maximum VALUE, not the count of values — a classic off-by-one trap!

Q17. Perform binary addition: 1011 + 1101 = ? - A) 11000 - B) 10110 - C) 11001 - D) 10111

Answer: A) 11000 1011 (11) + 1101 (13) = 24 = 11000₂. Working right to left with carries.

Q18. Which of the following is both an input and output device? - A) Keyboard - B) Printer - C) Touch Screen - D) Scanner

Answer: C) Touch Screen Touch screen takes input (touch) and displays output. Keyboard is input only, Printer is output only, Scanner is input only.

Q19. In 2’s complement representation using 8 bits, the range of integers is: - A) 0 to 255 - B) -127 to 127 - C) -128 to 127 - D) -128 to 128

Answer: C) -128 to 127 For n-bit 2’s complement: range is -2^(n-1) to 2^(n-1)-1 = -128 to 127 for 8 bits. Distractor D (-128 to 128) is wrong because +128 cannot be represented in 8-bit 2’s complement. Distractor A (0-255) is for unsigned 8-bit.

Q20. The hexadecimal number FF₁₆ in decimal is: - A) 256 - B) 255 - C) 240 - D) 160

Answer: B) 255 F×16¹ + F×16⁰ = 15×16 + 15×1 = 240 + 15 = 255. Distractor A (256) is 2⁸, a common confusion. Distractor C (240) is only 15×16, forgetting the second F.

## ASSIGNMENT — DAY 1

Part A: Number System Conversions (20 problems) 1. Convert to Binary: 73, 156, 200, 99, 512 2. Convert to Decimal: 11010110₂, 1F3₁₆, 745₈, 10000001₂ 3. Convert to Hexadecimal: 1024₁₀, 11110000₂, 777₈ 4. Convert to Octal: 255₁₀, 10101010₂, FF₁₆ 5. Convert 45.625₁₀ to Binary 6. Convert 0.1₂ to Decimal

Part B: Binary Arithmetic 7. Perform: 11011 + 10110 8. Perform: 10010 + 01111 9. Find 2’s complement of: 01101001, 11000011, 00001111 10. Subtract using 2’s complement: 25 - 13 (in 8-bit)

Part C: BCD 11. Write BCD for: 47, 96, 123, 255 12. Is 11011001 a valid BCD? Why or why not?

## HOME TASK — DAY 1

- Convert the following 25 decimal numbers to Binary, Octal, and Hexadecimal: 10, 23, 47, 64, 78, 99, 100, 127, 128, 150, 175, 192, 200, 210, 225, 240, 255, 256, 300, 400, 500, 512, 1000, 1024, 2048

- Practice 2’s complement for these 10 negative numbers (8-bit): -1, -5, -10, -25, -50, -64, -100, -127, -128, -3

- Read Chapter 1-2 of “Foundations of Computing” by Pradeep Sinha & Priti Sinha

- Self-test: Without looking at notes, write the 5 computer generations with their technologies

# DAY 2: Boolean Algebra, Logic Gates & Storage Systems

## THEORY (2 Hours)

### 2.1 Boolean Algebra

Boolean algebra is the mathematics of logic, dealing with only two values: TRUE (1) and FALSE (0). It is the foundation of digital computer design.

Basic Boolean Operations:

| Operation | Symbol | Expression | Meaning |
| --- | --- | --- | --- |
| AND | · or ∧ | A · B | Output 1 only when BOTH inputs are 1 |
| OR | + or ∨ | A + B | Output 1 when ANY input is 1 |
| NOT | ’ or ¬ or ‾ | A’ or Ā | Inverts: 0→1, 1→0 |

Boolean Algebra Laws:

| Law | AND Form | OR Form |
| --- | --- | --- |
| Identity | A · 1 = A | A + 0 = A |
| Null/Domination | A · 0 = 0 | A + 1 = 1 |
| Idempotent | A · A = A | A + A = A |
| Complement | A · A’ = 0 | A + A’ = 1 |
| Commutative | A · B = B · A | A + B = B + A |
| Associative | (A·B)·C = A·(B·C) | (A+B)+C = A+(B+C) |
| Distributive | A·(B+C) = A·B+A·C | A+(B·C) = (A+B)·(A+C) |
| Absorption | A·(A+B) = A | A+(A·B) = A |
| Double Negation | (A’)’ = A |  |

De Morgan’s Theorems — Very Important for C-CAT!

Theorem 1: (A · B)' = A' + B'
    "NOT of AND = OR of NOTs"

Theorem 2: (A + B)' = A' · B'
    "NOT of OR = AND of NOTs"

Example of simplification: Simplify: A·B + A·B’

= A·(B + B')     [Distributive law]
= A·(1)          [Complement law: B + B' = 1]
= A              [Identity law]

### 2.2 Logic Gates

Logic gates are the physical building blocks of digital circuits. Each gate implements a Boolean function.

1. AND Gate

Inputs: A, B    Output: Y = A · B
Truth Table:
A  B  | Y
0  0  | 0
0  1  | 0
1  0  | 0
1  1  | 1

Output is 1 ONLY when ALL inputs are 1.

2. OR Gate

Inputs: A, B    Output: Y = A + B
Truth Table:
A  B  | Y
0  0  | 0
0  1  | 1
1  0  | 1
1  1  | 1

Output is 1 when ANY input is 1.

3. NOT Gate (Inverter)

Input: A    Output: Y = A'
Truth Table:
A  | Y
0  | 1
1  | 0

Flips the input.

4. NAND Gate (NOT-AND) — Universal Gate

Output: Y = (A · B)'
Truth Table:
A  B  | Y
0  0  | 1
0  1  | 1
1  0  | 1
1  1  | 0

Output is 0 ONLY when ALL inputs are 1. (Opposite of AND)

5. NOR Gate (NOT-OR) — Universal Gate

Output: Y = (A + B)'
Truth Table:
A  B  | Y
0  0  | 1
0  1  | 0
1  0  | 0
1  1  | 0

Output is 1 ONLY when ALL inputs are 0. (Opposite of OR)

6. XOR Gate (Exclusive OR)

Output: Y = A ⊕ B = A'B + AB'
Truth Table:
A  B  | Y
0  0  | 0
0  1  | 1
1  0  | 1
1  1  | 0

Output is 1 when inputs are DIFFERENT.

7. XNOR Gate (Exclusive NOR)

Output: Y = (A ⊕ B)' = AB + A'B'
Truth Table:
A  B  | Y
0  0  | 1
0  1  | 0
1  0  | 0
1  1  | 1

Output is 1 when inputs are SAME.

C-CAT Tip: NAND and NOR are called Universal Gates because ANY other gate can be built using only NAND gates (or only NOR gates). This is frequently asked!

Implementing gates using only NAND: - NOT A = A NAND A - A AND B = (A NAND B) NAND (A NAND B) - A OR B = (A NAND A) NAND (B NAND B)

### 2.3 Half Adder and Full Adder

Half Adder: Adds two single-bit numbers

Inputs: A, B
Outputs: Sum = A ⊕ B (XOR), Carry = A · B (AND)

A  B  | Sum  Carry
0  0  |  0    0
0  1  |  1    0
1  0  |  1    0
1  1  |  0    1

Full Adder: Adds three single-bit numbers (including carry-in)

Inputs: A, B, Cin
Outputs: Sum = A ⊕ B ⊕ Cin, Cout = AB + BCin + ACin

A  B  Cin | Sum  Cout
0  0   0  |  0    0
0  0   1  |  1    0
0  1   0  |  1    0
0  1   1  |  0    1
1  0   0  |  1    0
1  0   1  |  0    1
1  1   0  |  0    1
1  1   1  |  1    1

### 2.4 Storage Systems

Primary Storage (Main Memory):

RAM (Random Access Memory) — Volatile - SRAM (Static RAM): Uses flip-flops, faster, expensive, used for Cache - DRAM (Dynamic RAM): Uses capacitors, needs refreshing, cheaper, used for Main Memory - DRAM is slower than SRAM but much cheaper and denser

ROM (Read Only Memory) — Non-volatile - ROM: Factory programmed, cannot be changed - PROM: Programmable ROM — can be programmed once by user - EPROM: Erasable PROM — erased using UV light, reprogrammable - EEPROM: Electrically Erasable PROM — erased electrically, byte-by-byte - Flash Memory: Type of EEPROM — erased in blocks, used in USB drives, SSDs, memory cards

Secondary Storage:

| Device | Type | Access | Speed | Capacity |
| --- | --- | --- | --- | --- |
| HDD | Magnetic | Sequential/Random | Moderate | Very High (TB) |
| SSD | Flash (Electronic) | Random | Fast | High (TB) |
| CD-ROM | Optical | Sequential | Slow | 700 MB |
| DVD | Optical | Sequential | Slow | 4.7 GB / 8.5 GB (DL) |
| Blu-ray | Optical | Sequential | Moderate | 25 GB / 50 GB (DL) |
| Magnetic Tape | Magnetic | Sequential ONLY | Very Slow | Very High (backup) |

Storage Capacity Units:

1 Bit = 0 or 1
1 Nibble = 4 bits
1 Byte = 8 bits
1 KB (Kilobyte) = 1024 bytes = 2¹⁰ bytes
1 MB (Megabyte) = 1024 KB = 2²⁰ bytes
1 GB (Gigabyte) = 1024 MB = 2³⁰ bytes
1 TB (Terabyte) = 1024 GB = 2⁴⁰ bytes
1 PB (Petabyte) = 1024 TB = 2⁵⁰ bytes

C-CAT Tip: 1 KB = 1024 bytes (NOT 1000). This is tested! Also, know the difference between SRAM and DRAM.

## MCQs — DAY 2 (20 Questions)

Q1. Simplify: A + A’B = ? - A) A + B - B) A - C) B - D) AB

Answer: A) A + B Using Boolean algebra: A + A’B = (A + A’)(A + B) [Distributive: A+(B·C) = (A+B)·(A+C)] = 1·(A+B) = A+B. Distractor B (just A) is wrong — the A’B term adds B’s contribution when A=0.

Q2. According to De Morgan’s theorem, (A + B)’ equals: - A) A’ + B’ - B) A’ · B’ - C) A · B - D) (A · B)’

Answer: B) A’ · B’ De Morgan’s 2nd theorem: complement of OR = AND of complements. Distractor A (A’+B’) is the complement of AND (De Morgan’s 1st theorem) — students often swap the two theorems!

Q3. Which gates are called Universal Gates? - A) AND and OR - B) NAND and NOR - C) XOR and XNOR - D) NOT and AND

Answer: B) NAND and NOR They can implement any Boolean function alone. AND/OR cannot implement NOT on their own.

Q4. The output of XOR gate when both inputs are 1 is: - A) 1 - B) 0 - C) Undefined - D) Depends on previous state

Answer: B) 0 XOR outputs 1 only when inputs are DIFFERENT. When both are 1, output is 0. Distractor A is the OR gate output — students confuse XOR with OR.

Q5. The truth table output for NAND gate with inputs A=1, B=0 is: - A) 0 - B) 1 - C) Undefined - D) Depends on gate

Answer: B) 1 NAND = NOT(AND). AND(1,0) = 0, NOT(0) = 1. NAND outputs 0 ONLY when both inputs are 1.

Q6. How many NAND gates are required to implement a NOT gate? - A) 1 - B) 2 - C) 3 - D) 4

Answer: A) 1 Connect both inputs of NAND to the same signal: A NAND A = (A·A)’ = A’. Only 1 NAND gate needed. Distractor B (2) is what’s needed for AND, not NOT.

Q7. The Sum output of a Half Adder is equivalent to: - A) AND gate - B) OR gate - C) XOR gate - D) NAND gate

Answer: C) XOR gate Sum = A ⊕ B (XOR). Carry = A · B (AND). Distractor B (OR) gives 1 for (1,1) but Sum should be 0 for (1,1) with carry 1.

Q8. SRAM is used for: - A) Main memory - B) Cache memory - C) Secondary storage - D) ROM

Answer: B) Cache memory SRAM is faster but more expensive, making it ideal for cache. DRAM is used for main memory (cheaper, denser). This is a frequently tested distinction.

Q9. Which type of ROM can be erased using ultraviolet light? - A) PROM - B) EPROM - C) EEPROM - D) Flash Memory

Answer: B) EPROM EPROM = Erasable PROM, erased by UV light. EEPROM is erased electrically. PROM can only be programmed once. Distractor C (EEPROM) is electrically erasable, not UV.

Q10. Simplify: A · (A + B) = ? - A) A + B - B) A · B - C) A - D) B

Answer: C) A This is the Absorption Law: A·(A+B) = A. Proof: A·A + A·B = A + A·B = A·(1+B) = A·1 = A. Distractor A (A+B) is wrong — the AND with A limits the result to just A.

Q11. 1 Megabyte equals: - A) 1000 KB - B) 1024 KB - C) 1000000 bytes - D) 1048576 bits

Answer: B) 1024 KB 1 MB = 1024 KB = 1024 × 1024 bytes = 1,048,576 bytes. Distractor A (1000 KB) is the decimal approximation, not the actual computer science value. Distractor D would be correct if it said bytes, but it says bits.

Q12. The main difference between SRAM and DRAM is: - A) SRAM is non-volatile, DRAM is volatile - B) SRAM uses capacitors, DRAM uses flip-flops - C) SRAM is faster and costlier, DRAM is slower and cheaper - D) SRAM needs refreshing, DRAM does not

Answer: C) SRAM is faster and costlier, DRAM is slower and cheaper Both are volatile (A is wrong). SRAM uses flip-flops, DRAM uses capacitors (B has them swapped — a classic trap!). DRAM needs refreshing, not SRAM (D is swapped).

Q13. The Boolean expression for XNOR gate is: - A) A ⊕ B - B) (A ⊕ B)’ - C) A’ + B’ - D) A · B

Answer: B) (A ⊕ B)’ XNOR is the complement of XOR. It equals AB + A’B’ (output 1 when inputs are SAME). Distractor A is XOR itself, not XNOR.

Q14. A Full Adder has how many inputs and outputs? - A) 2 inputs, 1 output - B) 2 inputs, 2 outputs - C) 3 inputs, 2 outputs - D) 3 inputs, 3 outputs

Answer: C) 3 inputs, 2 outputs Inputs: A, B, Carry-in. Outputs: Sum, Carry-out. A Half Adder has 2 inputs and 2 outputs (B is for Half Adder).

Q15. Which storage device uses sequential access ONLY? - A) Hard Disk - B) SSD - C) Magnetic Tape - D) CD-ROM

Answer: C) Magnetic Tape Tape can only be read sequentially (like a cassette — must fast-forward to reach data). HDD and CD-ROM support random access (though HDD is faster at it). SSD has true random access.

Q16. Evaluate: (1 + 0) · (1 + 1) · (0 + 1) = ? - A) 0 - B) 1 - C) Undefined - D) 2

Answer: B) 1 = 1 · 1 · 1 = 1. Remember: in Boolean algebra, 1+1 = 1 (not 2!). Distractor D (2) is wrong because Boolean has only 0 and 1.

Q17. Flash memory is a type of: - A) SRAM - B) DRAM - C) EEPROM - D) PROM

Answer: C) EEPROM Flash is a special type of EEPROM that erases in blocks rather than byte-by-byte. It’s electrically erasable and reprogrammable.

Q18. The complement of expression AB + CD using De Morgan’s theorem is: - A) A’B’ + C’D’ - B) (A’+B’)(C’+D’) - C) (A’+B’) + (C’+D’) - D) A’B’C’D’

Answer: B) (A’+B’)(C’+D’) Step 1: (AB + CD)’ = (AB)‘·(CD)’ [De Morgan’s on OR]. Step 2: = (A’+B’)·(C’+D’) [De Morgan’s on AND]. Distractor A applies De Morgan’s incorrectly by keeping the OR.

Q19. How many bytes are in 4 KB? - A) 4000 - B) 4096 - C) 4048 - D) 4080

Answer: B) 4096 4 KB = 4 × 1024 = 4096 bytes. Distractor A (4000) uses decimal 1000 instead of 1024.

Q20. The NOR gate output is 1 only when: - A) Both inputs are 1 - B) Both inputs are 0 - C) At least one input is 1 - D) Inputs are different

Answer: B) Both inputs are 0 NOR = NOT(OR). OR(0,0) = 0, NOT(0) = 1. For any other input combination, OR gives 1, so NOR gives 0. Distractor A describes AND gate, C describes OR gate, D describes XOR gate.

## ASSIGNMENT — DAY 2

Part A: Boolean Algebra (10 problems) 1. Simplify: A·B + A·B’ + A’·B 2. Simplify: (A + B)(A + B’) 3. Simplify: A’B’C + A’BC + AB’C + ABC 4. Apply De Morgan’s theorem to: (ABC)’ 5. Apply De Morgan’s theorem to: (A + B + C)’ 6. Prove: A + AB = A (Absorption law) 7. Simplify: (A + B)(A’ + C)(B + C) 8. Find complement of: AB’ + A’B

Part B: Logic Gate Analysis (5 problems) 9. Draw the circuit for: Y = AB + A’C using AND, OR, NOT gates 10. Implement AND gate using only NOR gates (draw the circuit) 11. If inputs to an XOR gate alternate between (0,0), (0,1), (1,0), (1,1), list all outputs 12. Given a circuit: A → NOT → input1 of AND; B → input2 of AND → output Y. Write the Boolean expression and truth table. 13. How many 2-input NAND gates are needed to build a 2-input OR gate?

Part C: Storage 14. Fill in the comparison table: SRAM vs DRAM vs ROM vs Flash (Speed, Volatility, Cost, Use) 15. Calculate: How many 700 MB CDs needed to store 4.2 GB of data?

## HOME TASK — DAY 2

- Draw truth tables for all 7 gates (AND, OR, NOT, NAND, NOR, XOR, XNOR)

- Simplify 5 Boolean expressions using De Morgan’s theorem:

  - (A·B·C)‘, (A+B+C)’, (AB+CD)‘, (A’B+AB’)‘, (A(B+C))’

- Practice implementing AND, OR, NOT gates using only NAND gates — draw circuits

- Memorize the storage hierarchy and capacity units

- Read Chapters 3-5 of “Foundations of Computing”

# DAY 3: Software, Operating Systems & Programming Concepts

## THEORY (2 Hours)

### 3.1 Software Classification

Software is a set of programs and instructions that tell the computer what to do.

System Software: Controls and manages computer hardware; provides a platform for application software - Operating System: Windows, Linux, macOS, Android, iOS - Language Translators: Assembler, Compiler, Interpreter - Device Drivers: Software that controls specific hardware (printer driver, graphics driver) - Utility Programs: Antivirus, disk defragmenter, file manager, backup tools

Application Software: Performs specific user tasks - General Purpose: MS Word, Excel, PowerPoint, Chrome, VLC - Special Purpose: Tally (accounting), AutoCAD (design), SPSS (statistics) - Custom Software: Built for specific organization’s needs (banking software, hospital management)

### 3.2 Programming Languages — Generations

| Generation | Type | Characteristics | Example |
| --- | --- | --- | --- |
| 1GL | Machine Language | Binary (0s and 1s), directly understood by CPU, hardware-dependent, very fast execution but extremely difficult to write | 10110000 01100001 |
| 2GL | Assembly Language | Mnemonics (ADD, MOV, SUB), needs Assembler to convert, hardware-dependent, slightly easier than 1GL | MOV AX, 5 |
| 3GL | High-Level Language | English-like, portable, needs Compiler/Interpreter | C, C++, Java, Python |
| 4GL | Very High-Level | Closer to natural language, database-oriented | SQL, MATLAB, R |
| 5GL | Natural Language | AI-based, problem-solving | Prolog, LISP, Mercury |

Low-level languages (1GL, 2GL): Close to hardware, fast, hardware-dependent High-level languages (3GL+): Close to human language, portable, slower (need translation)

### 3.3 Language Translators — Frequently tested in C-CAT!

Assembler - Translates Assembly language (2GL) → Machine language (1GL) - One-to-one translation (one assembly instruction = one machine instruction)

Compiler - Translates entire High-level program → Machine language AT ONCE - Creates an executable file (.exe) - Errors shown AFTER complete compilation - Faster execution (compiled once, run many times) - Examples: C compiler (gcc), C++ compiler (g++), Java compiler (javac)

Interpreter - Translates High-level program → Machine language LINE BY LINE - No executable file created - Errors shown immediately (stops at first error) - Slower execution (translates every time it runs) - Examples: Python, JavaScript, Ruby, PHP

Compiler vs Interpreter — Key Differences:

| Feature | Compiler | Interpreter |
| --- | --- | --- |
| Translation | Entire program at once | Line by line |
| Speed of execution | Faster (pre-compiled) | Slower (translates each time) |
| Executable file | Yes (.exe) | No |
| Error display | All errors after compilation | Stops at first error |
| Memory | More (stores object code) | Less |
| Debugging | Harder (all errors at once) | Easier (one error at a time) |
| Examples | C, C++, Java (partial) | Python, JavaScript, Ruby |

C-CAT Trap: Java uses BOTH — javac compiler creates bytecode (.class), then JVM interpreter/JIT executes it. If asked “Is Java compiled or interpreted?”, the answer is “Both” — it’s a hybrid approach.

### 3.4 Operating Systems

Definition: An OS is system software that manages computer hardware, software resources, and provides common services for computer programs. It acts as an intermediary between the user and the hardware.

Functions of OS: 1. Process Management: Creating, scheduling, and terminating processes 2. Memory Management: Allocating and deallocating memory to programs 3. File Management: Creating, deleting, organizing files and directories 4. I/O Management: Managing input/output devices and their drivers 5. Security: User authentication, access control, data protection 6. Networking: Managing network connections and communication

Types of Operating Systems:

| Type | Description | Example |
| --- | --- | --- |
| Batch OS | Jobs collected in batches, no user interaction during execution | Early IBM mainframes |
| Multiprogramming OS | Multiple programs in memory, CPU switches when one waits for I/O | IBM OS/360 |
| Multitasking/Time-sharing OS | Each program gets a small time slice (quantum), appears simultaneous | Unix, Windows, Linux |
| Real-time OS (RTOS) | Strict time deadlines, used in critical systems | VxWorks (satellites), RTLinux |
| Distributed OS | Multiple computers appear as single system to user | Google’s systems, Amoeba |
| Embedded OS | Built into devices, limited resources | Android (phones), FreeRTOS (IoT) |
| Network OS | Manages network resources, file sharing | Windows Server, Novell NetWare |

C-CAT Tip: Know the difference between Multiprogramming and Multitasking. Multiprogramming = multiple programs in memory, CPU switches only during I/O wait. Multitasking = CPU rapidly switches between programs using time slices (time quantum), giving the illusion of parallelism.

### 3.5 Algorithms and Flowcharts

Algorithm: A step-by-step procedure to solve a problem.

Characteristics of a good algorithm: 1. Input: Zero or more inputs 2. Output: At least one output 3. Definiteness: Each step clearly defined 4. Finiteness: Must terminate after finite steps 5. Effectiveness: Each step must be feasible

Example Algorithm: Find the largest of three numbers A, B, C

Step 1: START
Step 2: Read A, B, C
Step 3: Set MAX = A
Step 4: If B > MAX, then MAX = B
Step 5: If C > MAX, then MAX = C
Step 6: Print MAX
Step 7: STOP

Flowchart: A graphical representation of an algorithm using standard symbols.

Flowchart Symbols:

| Symbol | Shape | Purpose |
| --- | --- | --- |
| Terminal | Oval/Rounded rectangle | START and STOP |
| Process | Rectangle | Computation, assignment |
| Decision | Diamond | Condition (Yes/No or True/False) |
| Input/Output | Parallelogram | Read input, Print output |
| Flow Lines | Arrows | Direction of flow |
| Connector | Small circle | Connect parts of flowchart |
| Predefined Process | Rectangle with double vertical lines | Subroutine/function call |

### 3.6 Pseudocode

Pseudocode is an informal, human-readable description of an algorithm that uses the structural conventions of programming but is written in plain English.

Example: Check if a number is prime

BEGIN
    READ number
    SET isPrime = TRUE
    IF number <= 1 THEN
        SET isPrime = FALSE
    ELSE
        FOR i = 2 TO sqrt(number)
            IF number MOD i == 0 THEN
                SET isPrime = FALSE
                BREAK
            END IF
        END FOR
    END IF
    IF isPrime == TRUE THEN
        PRINT "Prime"
    ELSE
        PRINT "Not Prime"
    END IF
END

Example: Calculate factorial of N

BEGIN
    READ N
    SET factorial = 1
    FOR i = 1 TO N
        factorial = factorial * i
    END FOR
    PRINT factorial
END

### 3.7 Problem-Solving Approaches

Top-Down Design (Stepwise Refinement) - Break the problem into smaller sub-problems - Solve each sub-problem independently - Combine solutions - Example: Building a calculator → Break into: input module, add module, subtract module, display module

Modular Programming - Divide program into independent modules (functions) - Each module performs one specific task - Benefits: easier debugging, code reuse, team development

## MCQs — DAY 3 (20 Questions)

Q1. Which of the following is NOT system software? - A) Operating System - B) Compiler - C) Microsoft Word - D) Device Driver

Answer: C) Microsoft Word Word is application software (used for specific tasks). OS, compilers, and device drivers are all system software. This is basic but tested.

Q2. An interpreter translates: - A) The entire program at once into machine code - B) Assembly language to machine language - C) One line at a time into machine code - D) Machine code to high-level language

Answer: C) One line at a time into machine code A describes a compiler, B describes an assembler, D describes a decompiler (reverse engineering). Interpreter = line by line.

Q3. Which of the following is true about a Compiler? - A) It translates one line at a time - B) It creates an executable file - C) It stops at the first error found - D) It is slower in execution than an interpreter

Answer: B) It creates an executable file A is interpreter behavior. C is interpreter behavior (compiler shows all errors after full compilation). D is wrong — compiled code runs FASTER because it’s pre-translated.

Q4. Assembly language uses: - A) Binary codes - B) Mnemonics like ADD, MOV - C) English-like syntax - D) SQL queries

Answer: B) Mnemonics like ADD, MOV A is machine language. C describes high-level languages. Assembly is between machine and high-level — it uses short mnemonics that map to machine instructions.

Q5. In a Time-sharing Operating System: - A) Only one program runs at a time - B) CPU time is divided among multiple users/programs using time slices - C) Programs are grouped in batches - D) There are no interrupts

Answer: B) CPU time is divided among multiple users/programs using time slices A is single-tasking. C is batch processing. Time-sharing = rapid switching using time quantum, creating illusion of simultaneous execution.

Q6. The flowchart symbol for a decision/condition is: - A) Rectangle - B) Parallelogram - C) Diamond - D) Oval

Answer: C) Diamond Rectangle = Process, Parallelogram = Input/Output, Oval = Terminal (Start/Stop). Diamond has Yes/No or True/False branches.

Q7. Which programming language generation does Python belong to? - A) 2GL - B) 3GL - C) 4GL - D) 5GL

Answer: B) 3GL Python is a high-level language (3GL). Distractor C (4GL) — SQL and MATLAB are 4GL. Distractor D (5GL) — Prolog is 5GL. Students often think Python is 4GL because it’s “modern” — but generation is about the type, not the era.

Q8. The main difference between Multiprogramming and Multitasking is: - A) Multiprogramming uses multiple CPUs - B) Multitasking uses time slicing, multiprogramming switches only during I/O wait - C) They are the same thing - D) Multiprogramming is faster

Answer: B) Multitasking uses time slicing, multiprogramming switches only during I/O wait In multiprogramming, CPU switches to another program only when current one is waiting for I/O. In multitasking, CPU gives each program a fixed time slice and switches regardless. Distractor A describes multiprocessing (multiple CPUs).

Q9. Java is: - A) Purely compiled - B) Purely interpreted - C) Both compiled and interpreted - D) Neither compiled nor interpreted

Answer: C) Both compiled and interpreted Java source → compiled by javac → bytecode (.class file) → interpreted/JIT-compiled by JVM. This hybrid nature is a frequently asked concept.

Q10. An algorithm must have the property of: - A) Infinite steps - B) Finiteness - C) Ambiguous steps - D) No output

Answer: B) Finiteness An algorithm must terminate after a finite number of steps. Infinite loop is NOT an algorithm by definition. It must also have definiteness (clear steps), effectiveness, and at least one output.

Q11. Which of the following converts Assembly language to Machine language? - A) Compiler - B) Interpreter - C) Assembler - D) Linker

Answer: C) Assembler Compiler/Interpreter convert high-level languages. Assembler specifically converts assembly (2GL) to machine language (1GL). Distractor D (Linker) combines object files — it’s a different phase.

Q12. A Real-Time Operating System is used in: - A) Office computers - B) Satellite control systems - C) Personal laptops - D) Gaming consoles

Answer: B) Satellite control systems RTOS has strict time deadlines — critical for satellites, air traffic control, medical equipment, nuclear plants. Office PCs and laptops use general-purpose OS.

Q13. The flowchart symbol for input/output is: - A) Rectangle - B) Diamond - C) Parallelogram - D) Circle

Answer: C) Parallelogram Rectangle = Process, Diamond = Decision, Circle = Connector. The parallelogram is used for all I/O operations (READ, PRINT).

Q14. Which of the following is a characteristic of high-level languages? - A) Hardware-dependent - B) Portable across different computers - C) Uses binary code - D) Does not need translation

Answer: B) Portable across different computers High-level languages are machine-independent (portable). A and C describe low-level languages. D is wrong — all HLL need compilation/interpretation.

Q15. FORTRAN was primarily designed for: - A) Business applications - B) Scientific and mathematical computation - C) Web development - D) Database management

Answer: B) Scientific and mathematical computation FORTRAN = FORmula TRANslation — designed for scientific computing. COBOL was for business (A). Distractor C/D are modern applications.

Q16. An operating system acts as an intermediary between: - A) Compiler and Assembler - B) User and Hardware - C) RAM and ROM - D) Input and Output devices

Answer: B) User and Hardware The OS provides the interface between the user (and applications) and the underlying hardware. This is the fundamental definition of an OS.

Q17. Which of the following correctly orders programming language generations from lowest to highest level? - A) Assembly → Machine → High-level → 4GL - B) Machine → Assembly → High-level → 4GL - C) High-level → Assembly → Machine → 4GL - D) Machine → High-level → Assembly → 4GL

Answer: B) Machine → Assembly → High-level → 4GL Machine (1GL) is lowest level (binary), Assembly (2GL) uses mnemonics, High-level (3GL) uses English-like syntax, 4GL is even more abstract. Distractor A swaps Machine and Assembly.

Q18. What does BIOS stand for? - A) Basic Internal Operating System - B) Basic Input Output System - C) Binary Input Output System - D) Basic Integrated Operating Software

Answer: B) Basic Input Output System BIOS is firmware stored in ROM that initializes hardware during booting and provides runtime services. It’s the first software that runs when you turn on a computer.

Q19. The process of starting a computer is called: - A) Formatting - B) Booting - C) Compiling - D) Debugging

Answer: B) Booting Booting loads the OS from secondary storage to RAM. Cold boot = from power off. Warm boot = restart (Ctrl+Alt+Del). Formatting erases data, Compiling translates code, Debugging finds errors.

Q20. In top-down design approach: - A) The program is built from small modules upward - B) The problem is broken into smaller sub-problems - C) Only one module is created - D) Testing is done before coding

Answer: B) The problem is broken into smaller sub-problems Top-down = start with the overall problem and break it down. A describes bottom-up approach. This is a fundamental software engineering concept.

## ASSIGNMENT — DAY 3

Part A: Translator Comparison 1. Create a detailed comparison table: Assembler vs Compiler vs Interpreter (at least 8 parameters) 2. Give 3 examples of compiled languages and 3 examples of interpreted languages 3. Explain why Java is considered both compiled and interpreted (with diagram)

Part B: Flowcharts and Pseudocode Draw flowcharts AND write pseudocode for: 4. Find the largest of 3 numbers 5. Check if a number is even or odd 6. Calculate factorial of N 7. Print Fibonacci series up to N terms 8. Check if a number is prime 9. Find sum of digits of a number 10. Calculate simple interest

Part C: OS Concepts 11. List 5 functions of an Operating System with one-line explanations 12. Differentiate between Multiprogramming and Multitasking with examples 13. Give 3 real-world examples of Real-time Operating Systems

## HOME TASK — DAY 3

- Write pseudocode and draw flowcharts for 10 additional problems:

  - Reverse a number

  - Check Armstrong number

  - Find GCD of two numbers

  - Print multiplication table

  - Convert Celsius to Fahrenheit

  - Check leap year

  - Find area and perimeter of rectangle

  - Sum of first N natural numbers

  - Check if number is palindrome

  - Find power of a number (x^n)

- Read about compiler phases: Lexical Analysis → Syntax Analysis → Semantic Analysis → Code Generation → Code Optimization

- Revise ALL Day 1-3 topics — prepare for Day 4 (C Programming begins!)

- Self-test: List all 7 flowchart symbols with their shapes and purposes from memory

## DAY 1-3 REVISION CHECKLIST

Before moving to C Programming, ensure you can:

- Convert between all 4 number systems (Binary, Octal, Decimal, Hex) fluently

- Calculate 1’s and 2’s complement

- Perform binary arithmetic

- Write BCD for any decimal number

- Simplify Boolean expressions using all laws

- Apply De Morgan’s theorems

- Draw truth tables for all 7 logic gates

- Explain the memory hierarchy

- Differentiate SRAM vs DRAM vs ROM types

- Compare Compiler vs Interpreter vs Assembler

- List all OS types with characteristics

- Draw flowcharts for basic algorithms

- Write pseudocode for common problems

Module 1 Complete — 3 Days, 6 Hours Theory, 60 MCQs, 3 Assignments, 3 Home Tasks Next: Module 2 — C Programming (Days 4-7)