# DAY 2: Boolean Algebra, Logic Gates & Storage Systems

## THEORY (2 Hours)

### 2.1 Boolean Algebra

Boolean algebra is the mathematics of logic, dealing with only two values: TRUE (1) and FALSE (0). It is the foundation of digital computer design.

Basic Boolean Operations:

| Operation | Symbol | Expression | Meaning |
| --- | --- | --- | --- |
| AND | · or ∧ | A · B | Output 1 only when BOTH inputs are 1 |
| OR | + or ∨ | A + B | Output 1 when ANY input is 1 |
| NOT | ’ or ¬ or ‾ | A’ or Ā | Inverts: 0→1, 1→0 |

Boolean Algebra Laws:

| Law | AND Form | OR Form |
| --- | --- | --- |
| Identity | A · 1 = A | A + 0 = A |
| Null/Domination | A · 0 = 0 | A + 1 = 1 |
| Idempotent | A · A = A | A + A = A |
| Complement | A · A’ = 0 | A + A’ = 1 |
| Commutative | A · B = B · A | A + B = B + A |
| Associative | (A·B)·C = A·(B·C) | (A+B)+C = A+(B+C) |
| Distributive | A·(B+C) = A·B+A·C | A+(B·C) = (A+B)·(A+C) |
| Absorption | A·(A+B) = A | A+(A·B) = A |
| Double Negation | (A’)’ = A |  |

De Morgan’s Theorems — Very Important for C-CAT!

Theorem 1: (A · B)' = A' + B'
    "NOT of AND = OR of NOTs"

Theorem 2: (A + B)' = A' · B'
    "NOT of OR = AND of NOTs"

Example of simplification: Simplify: A·B + A·B’

= A·(B + B')     [Distributive law]
= A·(1)          [Complement law: B + B' = 1]
= A              [Identity law]
*** https://www.youtube.com/watch?v=gI-qXk7XojA

### 2.2 Logic Gates

Logic gates are the physical building blocks of digital circuits. Each gate implements a Boolean function.

1. AND Gate

Inputs: A, B    Output: Y = A · B
Truth Table:
A  B  | Y
0  0  | 0
0  1  | 0
1  0  | 0
1  1  | 1

Output is 1 ONLY when ALL inputs are 1.

2. OR Gate

Inputs: A, B    Output: Y = A + B
Truth Table:
A  B  | Y
0  0  | 0
0  1  | 1
1  0  | 1
1  1  | 1

Output is 1 when ANY input is 1.

3. NOT Gate (Inverter)

Input: A    Output: Y = A'
Truth Table:
A  | Y
0  | 1
1  | 0

Flips the input.

4. NAND Gate (NOT-AND) — Universal Gate

Output: Y = (A · B)'
Truth Table:
A  B  | Y
0  0  | 1
0  1  | 1
1  0  | 1
1  1  | 0

Output is 0 ONLY when ALL inputs are 1. (Opposite of AND)

5. NOR Gate (NOT-OR) — Universal Gate

Output: Y = (A + B)'
Truth Table:
A  B  | Y
0  0  | 1
0  1  | 0
1  0  | 0
1  1  | 0

Output is 1 ONLY when ALL inputs are 0. (Opposite of OR)

6. XOR Gate (Exclusive OR)

Output: Y = A ⊕ B = A'B + AB'
Truth Table:
A  B  | Y
0  0  | 0
0  1  | 1
1  0  | 1
1  1  | 0

Output is 1 when inputs are DIFFERENT.

7. XNOR Gate (Exclusive NOR)

Output: Y = (A ⊕ B)' = AB + A'B'
Truth Table:
A  B  | Y
0  0  | 1
0  1  | 0
1  0  | 0
1  1  | 1

Output is 1 when inputs are SAME.

C-CAT Tip: NAND and NOR are called Universal Gates because ANY other gate can be built using only NAND gates (or only NOR gates). This is frequently asked!

Implementing gates using only NAND: - NOT A = A NAND A - A AND B = (A NAND B) NAND (A NAND B) - A OR B = (A NAND A) NAND (B NAND B)

*** https://www.youtube.com/watch?v=sTu3LwpF6XI

### 2.3 Half Adder and Full Adder

Half Adder: Adds two single-bit numbers

Inputs: A, B
Outputs: Sum = A ⊕ B (XOR), Carry = A · B (AND)

A  B  | Sum  Carry
0  0  |  0    0
0  1  |  1    0
1  0  |  1    0
1  1  |  0    1

Full Adder: Adds three single-bit numbers (including carry-in)

Inputs: A, B, Cin
Outputs: Sum = A ⊕ B ⊕ Cin, Cout = AB + BCin + ACin

A  B  Cin | Sum  Cout
0  0   0  |  0    0
0  0   1  |  1    0
0  1   0  |  1    0
0  1   1  |  0    1
1  0   0  |  1    0
1  0   1  |  0    1
1  1   0  |  0    1
1  1   1  |  1    1

### 2.4 Storage Systems

Primary Storage (Main Memory):

RAM (Random Access Memory) — Volatile - SRAM (Static RAM): Uses flip-flops, faster, expensive, used for Cache - DRAM (Dynamic RAM): Uses capacitors, needs refreshing, cheaper, used for Main Memory - DRAM is slower than SRAM but much cheaper and denser

ROM (Read Only Memory) — Non-volatile - ROM: Factory programmed, cannot be changed - PROM: Programmable ROM — can be programmed once by user - EPROM: Erasable PROM — erased using UV light, reprogrammable - EEPROM: Electrically Erasable PROM — erased electrically, byte-by-byte - Flash Memory: Type of EEPROM — erased in blocks, used in USB drives, SSDs, memory cards

Secondary Storage:

| Device | Type | Access | Speed | Capacity |
| --- | --- | --- | --- | --- |
| HDD | Magnetic | Sequential/Random | Moderate | Very High (TB) |
| SSD | Flash (Electronic) | Random | Fast | High (TB) |
| CD-ROM | Optical | Sequential | Slow | 700 MB |
| DVD | Optical | Sequential | Slow | 4.7 GB / 8.5 GB (DL) |
| Blu-ray | Optical | Sequential | Moderate | 25 GB / 50 GB (DL) |
| Magnetic Tape | Magnetic | Sequential ONLY | Very Slow | Very High (backup) |

Storage Capacity Units:

1 Bit = 0 or 1
1 Nibble = 4 bits
1 Byte = 8 bits
1 KB (Kilobyte) = 1024 bytes = 2¹⁰ bytes
1 MB (Megabyte) = 1024 KB = 2²⁰ bytes
1 GB (Gigabyte) = 1024 MB = 2³⁰ bytes
1 TB (Terabyte) = 1024 GB = 2⁴⁰ bytes
1 PB (Petabyte) = 1024 TB = 2⁵⁰ bytes

C-CAT Tip: 1 KB = 1024 bytes (NOT 1000). This is tested! Also, know the difference between SRAM and DRAM.

***https://www.youtube.com/watch?v=n6uPALWAyxc

## MCQs — DAY 2 (20 Questions)

Q1. Simplify: A + A’B = ? - A) A + B - B) A - C) B - D) AB

Answer: A) A + B Using Boolean algebra: A + A’B = (A + A’)(A + B) [Distributive: A+(B·C) = (A+B)·(A+C)] = 1·(A+B) = A+B. Distractor B (just A) is wrong — the A’B term adds B’s contribution when A=0.

Q2. According to De Morgan’s theorem, (A + B)’ equals: - A) A’ + B’ - B) A’ · B’ - C) A · B - D) (A · B)’

Answer: B) A’ · B’ De Morgan’s 2nd theorem: complement of OR = AND of complements. Distractor A (A’+B’) is the complement of AND (De Morgan’s 1st theorem) — students often swap the two theorems!

Q3. Which gates are called Universal Gates? - A) AND and OR - B) NAND and NOR - C) XOR and XNOR - D) NOT and AND

Answer: B) NAND and NOR They can implement any Boolean function alone. AND/OR cannot implement NOT on their own.

Q4. The output of XOR gate when both inputs are 1 is: - A) 1 - B) 0 - C) Undefined - D) Depends on previous state

Answer: B) 0 XOR outputs 1 only when inputs are DIFFERENT. When both are 1, output is 0. Distractor A is the OR gate output — students confuse XOR with OR.

Q5. The truth table output for NAND gate with inputs A=1, B=0 is: - A) 0 - B) 1 - C) Undefined - D) Depends on gate

Answer: B) 1 NAND = NOT(AND). AND(1,0) = 0, NOT(0) = 1. NAND outputs 0 ONLY when both inputs are 1.

Q6. How many NAND gates are required to implement a NOT gate? - A) 1 - B) 2 - C) 3 - D) 4

Answer: A) 1 Connect both inputs of NAND to the same signal: A NAND A = (A·A)’ = A’. Only 1 NAND gate needed. Distractor B (2) is what’s needed for AND, not NOT.

Q7. The Sum output of a Half Adder is equivalent to: - A) AND gate - B) OR gate - C) XOR gate - D) NAND gate

Answer: C) XOR gate Sum = A ⊕ B (XOR). Carry = A · B (AND). Distractor B (OR) gives 1 for (1,1) but Sum should be 0 for (1,1) with carry 1.

Q8. SRAM is used for: - A) Main memory - B) Cache memory - C) Secondary storage - D) ROM

Answer: B) Cache memory SRAM is faster but more expensive, making it ideal for cache. DRAM is used for main memory (cheaper, denser). This is a frequently tested distinction.

Q9. Which type of ROM can be erased using ultraviolet light? - A) PROM - B) EPROM - C) EEPROM - D) Flash Memory

Answer: B) EPROM EPROM = Erasable PROM, erased by UV light. EEPROM is erased electrically. PROM can only be programmed once. Distractor C (EEPROM) is electrically erasable, not UV.

Q10. Simplify: A · (A + B) = ? - A) A + B - B) A · B - C) A - D) B

Answer: C) A This is the Absorption Law: A·(A+B) = A. Proof: A·A + A·B = A + A·B = A·(1+B) = A·1 = A. Distractor A (A+B) is wrong — the AND with A limits the result to just A.

Q11. 1 Megabyte equals: - A) 1000 KB - B) 1024 KB - C) 1000000 bytes - D) 1048576 bits

Answer: B) 1024 KB 1 MB = 1024 KB = 1024 × 1024 bytes = 1,048,576 bytes. Distractor A (1000 KB) is the decimal approximation, not the actual computer science value. Distractor D would be correct if it said bytes, but it says bits.

Q12. The main difference between SRAM and DRAM is: - A) SRAM is non-volatile, DRAM is volatile - B) SRAM uses capacitors, DRAM uses flip-flops - C) SRAM is faster and costlier, DRAM is slower and cheaper - D) SRAM needs refreshing, DRAM does not

Answer: C) SRAM is faster and costlier, DRAM is slower and cheaper Both are volatile (A is wrong). SRAM uses flip-flops, DRAM uses capacitors (B has them swapped — a classic trap!). DRAM needs refreshing, not SRAM (D is swapped).

Q13. The Boolean expression for XNOR gate is: - A) A ⊕ B - B) (A ⊕ B)’ - C) A’ + B’ - D) A · B

Answer: B) (A ⊕ B)’ XNOR is the complement of XOR. It equals AB + A’B’ (output 1 when inputs are SAME). Distractor A is XOR itself, not XNOR.

Q14. A Full Adder has how many inputs and outputs? - A) 2 inputs, 1 output - B) 2 inputs, 2 outputs - C) 3 inputs, 2 outputs - D) 3 inputs, 3 outputs

Answer: C) 3 inputs, 2 outputs Inputs: A, B, Carry-in. Outputs: Sum, Carry-out. A Half Adder has 2 inputs and 2 outputs (B is for Half Adder).

Q15. Which storage device uses sequential access ONLY? - A) Hard Disk - B) SSD - C) Magnetic Tape - D) CD-ROM

Answer: C) Magnetic Tape Tape can only be read sequentially (like a cassette — must fast-forward to reach data). HDD and CD-ROM support random access (though HDD is faster at it). SSD has true random access.

Q16. Evaluate: (1 + 0) · (1 + 1) · (0 + 1) = ? - A) 0 - B) 1 - C) Undefined - D) 2

Answer: B) 1 = 1 · 1 · 1 = 1. Remember: in Boolean algebra, 1+1 = 1 (not 2!). Distractor D (2) is wrong because Boolean has only 0 and 1.

Q17. Flash memory is a type of: - A) SRAM - B) DRAM - C) EEPROM - D) PROM

Answer: C) EEPROM Flash is a special type of EEPROM that erases in blocks rather than byte-by-byte. It’s electrically erasable and reprogrammable.

Q18. The complement of expression AB + CD using De Morgan’s theorem is: - A) A’B’ + C’D’ - B) (A’+B’)(C’+D’) - C) (A’+B’) + (C’+D’) - D) A’B’C’D’

Answer: B) (A’+B’)(C’+D’) Step 1: (AB + CD)’ = (AB)‘·(CD)’ [De Morgan’s on OR]. Step 2: = (A’+B’)·(C’+D’) [De Morgan’s on AND]. Distractor A applies De Morgan’s incorrectly by keeping the OR.

Q19. How many bytes are in 4 KB? - A) 4000 - B) 4096 - C) 4048 - D) 4080

Answer: B) 4096 4 KB = 4 × 1024 = 4096 bytes. Distractor A (4000) uses decimal 1000 instead of 1024.

Q20. The NOR gate output is 1 only when: - A) Both inputs are 1 - B) Both inputs are 0 - C) At least one input is 1 - D) Inputs are different

Answer: B) Both inputs are 0 NOR = NOT(OR). OR(0,0) = 0, NOT(0) = 1. For any other input combination, OR gives 1, so NOR gives 0. Distractor A describes AND gate, C describes OR gate, D describes XOR gate.

## ASSIGNMENT — DAY 2

Part A: Boolean Algebra (10 problems) 1. Simplify: A·B + A·B’ + A’·B 2. Simplify: (A + B)(A + B’) 3. Simplify: A’B’C + A’BC + AB’C + ABC 4. Apply De Morgan’s theorem to: (ABC)’ 5. Apply De Morgan’s theorem to: (A + B + C)’ 6. Prove: A + AB = A (Absorption law) 7. Simplify: (A + B)(A’ + C)(B + C) 8. Find complement of: AB’ + A’B

Part B: Logic Gate Analysis (5 problems) 9. Draw the circuit for: Y = AB + A’C using AND, OR, NOT gates 10. Implement AND gate using only NOR gates (draw the circuit) 11. If inputs to an XOR gate alternate between (0,0), (0,1), (1,0), (1,1), list all outputs 12. Given a circuit: A → NOT → input1 of AND; B → input2 of AND → output Y. Write the Boolean expression and truth table. 13. How many 2-input NAND gates are needed to build a 2-input OR gate?

Part C: Storage 14. Fill in the comparison table: SRAM vs DRAM vs ROM vs Flash (Speed, Volatility, Cost, Use) 15. Calculate: How many 700 MB CDs needed to store 4.2 GB of data?

## HOME TASK — DAY 2

- Draw truth tables for all 7 gates (AND, OR, NOT, NAND, NOR, XOR, XNOR)

- Simplify 5 Boolean expressions using De Morgan’s theorem:

  - (A·B·C)‘, (A+B+C)’, (AB+CD)‘, (A’B+AB’)‘, (A(B+C))’

- Practice implementing AND, OR, NOT gates using only NAND gates — draw circuits

- Memorize the storage hierarchy and capacity units

- Read Chapters 3-5 of “Foundations of Computing”